# Tab 1

## I would like to request a programme change from CSE to CS due to academic, personal, and professional reasons. I faced significant mental stress and family-related pressure which negatively affected m

## Assumptions

The following assumptions explain the logic and standard conditions used to design the "Animal Kingdom" tribal network.

Hardware & Physical Connections

- The Hub Router: We assume that Router Elephant acts as the central hub. It requires multiple Serial ports because it physically connects to three different tribes (Lion, Dog, and Cat) simultaneously.

- Distance Simulation: We used Serial DCE cables for all connections between routers. This assumes the tribes are far apart geographically (WAN connections), even though they are logically close in the topology.

- Standard Cabling: We assume standard Ethernet Straight through cables are used for all local connections PCs to Switches, and all devices have stable power and physical security.

IP Addressing Rules

- VLSM Efficiency: The IP addressing scheme uses Variable Length Subnet Masking (VLSM) to minimize IP wastage. We assume the allotted subnets (Dog: /21, Cat: /22, Elephant: /25, Lion: /28) are sufficient for future growth within the defined "Max User" limits.

- Link Conservation: All connections between routers (WAN links) use a prefix of /30. This provides exactly 2 usable IP addresses per link, ensuring no IPs are wasted on the cables connecting the buildings.

- The Chief Rule: We assume that the Tribal Chiefs (Special Computers) and Servers must have Static IPs. This ensures they can always be found at the same address for security logs and administration. Regular tribe members use dynamic (DHCP) IPs.

Routing & Traffic Flow

- Protocol Selection (RIPv2): We chose RIP Version 2 as the routing protocol. Even though the network simulates long distances physically, it is logically small with only 4 routers. RIPv2 is simple, reliable for this size, and supports our specific subnets VLSM.

- No Backup Paths: The design assumes a single active path between tribes. If a cable is cut, connectivity is lost. No redundant backup routers were included in this specific simulation.

Network Services

- Centralized Management: We assume the Lion Tribe is the Capital. It hosts all critical servers WEB, DNS, Email, DHCP. If the link to the Lion tribe goes down, the other tribes lose these services.

- DHCP Relay: Because the DHCP server is far away (in the Lion tribe), we assume that the routers for Dog, Cat, and Elephant must act as Relay Agents. This is why the ip helper address command is required on their gateway interfaces.

- Identity: All users rely on the single domain animals.com for email and web services

| Name | Host | Host + 2 | Block Size | Host Bit | Network |
| --- | --- | --- | --- | --- | --- |
| Dog | 1200 | 1202 | 211=2048 | 11 | 21 |
| Cat | 800 | 802 | 210=1024 | 10 | 22 |
| Elephant | 100 | 102 | 27=128 | 7 | 25 |
| Lion | 7 | 9 | 24=16 | 4 | 28 |

## VLSM TREE

## IP Address Table

| Building | Host | Network Address | Subnet Mask | Usable Range | Default Gateway | Servers |
| --- | --- | --- | --- | --- | --- | --- |
| Dog | 1200 | 172.16.0.0 | 255.255.248.0 | 172.16.0.2 - 172.16.7.253 | 172.16.0.1 | DNS 172.16.12.131 |
| Cat | 800 | 172.16.8.0 | 255.255.252.0 | 172.16.8.2 - 172.16.11.253 | 172.16.8.1 | WEB 172.16.12.130 |
| Elephant | 100 | 172.16.12.0 | 255.255.255.128 | 172.16.12.2 - 172.16.12.125 | 172.16.12.1 | Email 172.16.12.132 |
| Lion | 7 | 172.16.12.128 | 255.255.255.240 | 172.16.12.130 - 172.16.12.142 | 172.16.12.129 | DHCP 172.16.12.133 |

## Server IP Address Table

| Server Name | Manual IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Web Server | 172.16.12.130 | 255.255.255.240 | 172.16.12.129 |
| DNS Server | 172.16.12.131 | 255.255.255.240 | 172.16.12.129 |
| Email Server | 172.16.12.132 | 255.255.255.240 | 172.16.12.129 |
| DHCP Server | 172.16.12.133 | 255.255.255.240 | 172.16.12.129 |

## Special Computer IP Address

| Name | Super Computer IP | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Dog | 172.16.7.254 | 255.255.248.0 | 172.16.0.1 |
| Cat | 172.16.11.254 | 255.255.252.0 | 172.16.8.1 |
| Elephant | 172.16.12.126 | 255.255.255.128 | 172.16.12.1 |
| Lion | 172.16.12.142 | 255.255.255.240 | 172.16.12.129 |

## Inter Router (WAN) Link Table

| Segment | Network Address | Mask | Router 1 IP | Router 2 IP |
| --- | --- | --- | --- | --- |
| Lion-Elephant | 10.0.1.0 | 255.255.255.252 | 10.0.1.1 (Lion) | 10.0.1.2 (Elephant) |
| Elephant-Dog | 10.0.2.0 | 255.255.255.252 | 10.0.2.1 (Elephant) | 10.0.2.2 (Dog) |
| Elephant-Cat | 10.0.3.0 | 255.255.255.252 | 10.0.3.1 (Elephant) | 10.0.3.2 (Cat) |

DHCP Server:

- Dog Pool:

  - Default Gateway: 172.16.0.1

  - DNS Server: 172.16.12.131

  - Start IP: 172.16.0.2

  - Subnet Mask: 255.255.248.0

  - Max User: 1200

- Cat Pool:

  - Default Gateway: 172.16.8.1

  - DNS Server: 172.16.12.131

  - Start IP: 172.16.8.2

  - Subnet Mask: 255.255.252.0

- Elephant Pool:

  - Default Gateway: 172.16.12.1

  - DNS Server: 172.16.12.131

  - Start IP: 172.16.12.2

  - Subnet Mask: 255.255.255.128

Web Server:

- IP Address: 172.16.12.130 (Static)

- Edit: index.html

<html>

<center><font size='+2' color='blue'>Welcome to the Animal Kingdom! </font></center>

</html>

DNS Server:

- Domain Name: www.animals.com

- Type: A Type

- Address: 172.16.12.130

Email Server:

- Domain Name: animals.com

- Users:

- lionking Pass: 1234

- dogchief Pass: 1234

- elephantchief Pass: 1234

- catchief Pass: 1234

## Commands

Lion Router:

Router>en

Router#conf t

Router(config)#hostname Router_Lion

Router_Lion(config)#interface GigabitEthernet0/0/0

Router_Lion(config-if)#ip address 172.16.12.129 255.255.255.240

Router_Lion(config-if)#no shutdown

Router_Lion(config-if)#exit

Router_Lion(config)#interface Serial0/1/0

Router_Lion(config-if)#ip address 10.0.1.1 255.255.255.252

Router_Lion(config-if)#clock rate 64000

Router_Lion(config-if)#no shutdown

Router_Lion(config-if)#exit

Router_Lion(config)#ip route 0.0.0.0 0.0.0.0  10.0.1.2

Router_Lion(config)#ip route 172.16.0.0 255.255.248.0 10.0.1.2

Router_Lion(config)#ip route 172.16.8.0 255.255.252.0 10.0.1.2

Router_Lion(config)#ip route 172.16.12.0 255.255.255.128 10.0.1.2

Router_Lion(config)#ip route 10.0.2.0 255.255.255.252 10.0.1.2

Router_Lion(config)#ip route 10.0.3.0 255.255.255.252 10.0.1.2

Router_Lion(config)#exit

Elephant Router:

Router>en

Router#conf t

Router(config)#hostname Router_Elephant

Router_Elephant(config)#interface GigabitEthernet0/0/0

Router_Elephant(config-if)#ip address 172.16.12.1 255.255.255.128

Router_Elephant(config-if)#ip helper-address 172.16.12.133

Router_Elephant(config-if)#no shutdown

Router_Elephant(config-if)#exit

Router_Elephant(config)#interface Serial0/1/0

Router_Elephant(config-if)#ip address 10.0.1.2 255.255.255.252

Router_Elephant(config-if)#no shutdown

Router_Elephant(config-if)#exit

Router_Elephant(config)#interface Serial0/1/1

Router_Elephant(config-if)#ip address 10.0.2.1 255.255.255.252

Router_Elephant(config-if)#clock rate 64000

Router_Elephant(config-if)#no shutdown

Router_Elephant(config-if)#exit

Router_Elephant(config)#interface Serial0/2/0

Router_Elephant(config-if)#ip address 10.0.3.1 255.255.255.252

Router_Elephant(config-if)#clock rate 64000

Router_Elephant(config-if)#no shutdown

Router_Elephant(config-if)#exit

Router_Elephant(config)#router rip

Router_Elephant(config-router)#version 2

Router_Elephant(config-router)#no auto-summary

Router_Elephant(config-router)#network 172.16.12.0

Router_Elephant(config-router)#network 10.0.1.0

Router_Elephant(config-router)#network 10.0.2.0

Router_Elephant(config-router)#network 10.0.3.0

Router_Elephant(config-router)#passive-interface GigabitEthernet0/0/0

Router_Elephant(config-router)#exit

Dog Router:

Router>en

Router#conf t

Router(config)#hostname Router_Dog

Router_Dog(config)#interface GigabitEthernet0/0/0

Router_Dog(config-if)#ip address 172.16.0.1 255.255.248.0

Router_Dog(config-if)#ip helper-address 172.16.12.133

Router_Dog(config-if)#no shutdown

Router_Dog(config-if)#exit

Router_Dog(config)#interface Serial0/1/0

Router_Dog(config-if)#ip address 10.0.2.2 255.255.255.252

Router_Dog(config-if)#no shutdown

Router_Dog(config-if)#exit

Router_Dog(config)#router rip

Router_Dog(config-router)#version 2

Router_Dog(config-router)#no auto-summary

Router_Dog(config-router)#network 172.16.0.0

Router_Dog(config-router)#network 10.0.2.0

Router_Dog(config-router)#passive-interface GigabitEthernet0/0/0

Router_Dog(config-router)#exit

Cat Router:

Router>en

Router#conf t

Router(config)#hostname Router_Cat

Router_Cat(config)#interface GigabitEthernet0/0/0

Router_Cat(config-if)#ip address 172.16.8.1 255.255.252.0

Router_Cat(config-if)#ip helper-address 172.16.12.133

Router_Cat(config-if)#no shutdown

Router_Cat(config-if)#exit

Router_Cat(config)#interface Serial0/1/0

Router_Cat(config-if)#ip address 10.0.3.2 255.255.255.252

Router_Cat(config-if)#no shutdown

Router_Cat(config-if)#exit

Router_Cat(config)#router rip

Router_Cat(config-router)#version 2

Router_Cat(config-router)#no auto-summary

Router_Cat(config-router)#network 172.16.8.0

Router_Cat(config-router)#network 10.0.3.0

Router_Cat(config-router)#passive-interface GigabitEthernet0/0/0

Router_Cat(config-router)#exit